The Fidelity–Utility Paradox in Surrogate-Based Reinforcement Learning: A Role-Separated Hybrid Methodology in an HVAC Control Case Study

Aksultan Mukhanbet 1, Almaz Sapargali 2,1,*, Serik Aibagarov 2,1, Beimbet Daribayev 3, Shona Shinassylov 2,1 and Paulo Trigo 4

1 Al-Farabi Kazakh National University, Almaty, Kazakhstan

2 LLP Digit Alem, Almaty, Kazakhstan

3 Shakarim University, Semey, Abai Region, Kazakhstan

4 Instituto Superior de Engenharia de Lisboa (ISEL), Lisbon, Portugal

* Correspondence: almaz.sapargali2001@gmail.com

Abstract

Deep reinforcement-learning (RL) controllers for HVAC are often trained against fast neural-network surrogates, under the assumption that a more accurate surrogate provides a better training environment. We test this assumption on the BOPTEST bestest_air testcase and report a negative result, the fidelity–utility paradox: the surrogate with lower predictive error can be the worse environment for policy-gradient search. A grey-box RC surrogate with a neural residual heat-flow head (GB) attains a 24-hour rollout RMSE of 0.644 °C versus 1.557 °C for a black-box surrogate (BB), yet a policy trained directly on GB collapses on every seed: its BOPTEST maintenance score is ms = 1.07/1.13 peak/typical (lower is better; ms > 1 is unusable, N = 3), with comfort violation > 75%, whereas the weaker BB trains a usable controller (ms = 0.073/0.095). Retraining BB at finer resolution makes it more accurate yet unusable (ms > 1.1), showing that the driver is temporal resolution and the coarse per-step increment, not model class alone. We then formulate a role-separated hybrid methodology: BB supplies smooth rollout dynamics, while a frozen GB acts only as a per-step model-disagreement reward censor. This role separation gives the best cross-window robustness (ms = 0.041, sub-5% violation) at ~85× live-simulator throughput, with a controller-family-specific censor weight. Across three hydronic transfer cases, recalibration transfers but the frozen controller does not, indicating a component-level rather than universal transfer boundary.

Keywords: reinforcement learning; HVAC control; surrogate model; physics-informed calibration; transfer learning; BOPTEST